# Assignment -2

Andrew John

18BEC1278

- Develop an "Automatic garage door opening system". Use an Ultrasonic sensor to detect if there is a vehicle in front of the garage. If any vehicle is detected, open the garage door (rotate the servo motor) for some time and close it.

CODE:

#include<Servo.h>

Servo s;

int trig = 8;

int echo = 9;

int getDist()

{

digitalWrite(trig,LOW);

digitalWrite(trig,HIGH);

delayMicroseconds(10);

digitalWrite(trig,LOW);

float dur = pulseIn(echo,HIGH);

float dis = (dur*0.034)/2;

return dis;

}

//Open the Door

void open()

{

for(int i=0;i<=180;i++){

s.write(i);

delay(100);}

}

//Close the Door

void close()

{

for(int j=180;j>=0;j--){

s.write(j);

delay(10);

}

}

//Initializing the Pins

void setup()

{

s.attach(4);

pinMode(trig, OUTPUT);

pinMode(echo, INPUT);

Serial.begin(9600);

}

void loop()

{

int dis = getDist();

Serial.print("Distance in cm: ");

Serial.println(dis);

if(dis < 50)

{

Serial.println("Distance is lesser than 50cm");

Serial.println("---Opening the Garage Door---");

open();

Serial.println("---Waiting for 5 secs---");

delay(5000);

Serial.println("---Closing the Garage Door---");

close();

}

else

{

Serial.println("Distance is greater than 50cm");

s.write(0);

}

Serial.println("---Delay of 5 secs---");

delay(5000);

}

CIRCUIT DESIGN:

OUTPUTS:

- Opening garage door for distance greater than 50cm.

Fig-1: Opening the Garage Door

Fig-2: Waiting for 5 seconds

Fig-3: Closing the Garage Door

Fig-4: Delay of 5 seconds

- Closing garage door for distance lesser than 50cm